Qualification: Advanced Diploma of Information Technology

STUDENT NUMBER: 218266770 Khoza M.J

SUBJECT NAME: COMPUTER NETWORKS IVA

SUBJECT CODE: PNA117V

LECTURE NAME: EM Letsoalo

DEPARTMENT NAME: IT

Table of content

Vlan and Ports Planning…………………………………………………………………… page3

IPV6 address Planning………………………………………………………………………. page4 to 8

IPv6 Address Assignment……………………………………………………………………page9 to 11

Network Topology………………………………………………………………………………page12

Show Vlan Brief………………………………………………………………………………. page13 to 17

Show Interface Vlan………………………………………………………………………. page18 to 21

Show Port Interface………………………………………………………………………...page22 to 25

Configuration Dumps………………………………………………………………………. page26 to 31

Show Ipv6 Route………………………………………………………………………………page32 to 41

Frame Relay……………………………………………………………………………………. page42

ACLs……………………………………………………………………………………………. page43 to 44

Ping Results………………………………………………………………………………. page 45 to 46

VLAN and Ports Planning

| Vlan ID | Vlan Name | Ports |
| --- | --- | --- |
| 25 | PNA117V | G1/0/1-10 |
| 35 | NWP117V | G1/0/11-20 |
| 35 | Data Center VLAN1 | Fa0/1-10 |
| 50 | Data Center VLAN2 | Fa0/11-20 |
| 70 | Remote Branch | Fa0/1-20 |
| 80 | Recovery site | Fa0/1-20 |

IPV6 address Planning

PNA117V Network

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:1::/64 | 2025:ACE:AD:1::1/64 | 2025:ACE:AD:1::2/64 |

NWP117V

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:2::/64 | 2025:ACE:AD:2::1/64 | 2025:ACE:AD:2::2/64 |

Data Center Network

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:3::/64 | 2025:ACE:AD:3::1/64 | 2025:ACE:AD:3::2/64 |
| 2025:ACE:AD:4::/64 | 2025:ACE:AD:4::1/64 | 2025:ACE:AD:4::2/64 |

Remote Branch Network

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:5::/64 | 2025:ACE:AD:5::1/64 | 2025:ACE:AD:5::2/64 |

Recovery Site Network

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:6::/64 | 2025:ACE:AD:6::1/64 | 2025:ACE:AD:6::2/64 |

WAN 1

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:A::/64 | 2025:ACE:AD:A::1/64 | 2025:ACE:AD:A::2/64 |

WAN 2

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:B::/64 | 2025:ACE:AD:B::1/64 | 2025:ACE:AD:B::2/64 |

WAN 3

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:C::/64 | 2025:ACE:AD:C::1/64 | 2025:ACE:AD:C::2/64 |

WAN 4

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:D::/64 | 2025:ACE:AD:D::1/64 | 2025:ACE:AD:D::2/64 |

WAN 5

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:E::/64 | 2025:ACE:AD:E::1/64 | 2025:ACE:AD:E::2/64 |

WAN 6

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:F::/64 | 2025:ACE:AD:F::1/64 | 2025:ACE:AD:F::2/64 |

WAN 7

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:10::/64 | 2025:ACE:AD:10::1/64 | 2025:ACE:AD:10::2/64 |

WAN 8

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:11::/64 | 2025:ACE:AD:11::1/64 | 2025:ACE:AD:11::2/64 |

WAN 9

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:12::/64 | 2025:ACE:AD:12::1/64 | 2025:ACE:AD:12::2/64 |

WAN 10

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:13::/64 | 2025:ACE:AD:13::1/64 | 2025:ACE:AD:13::2/64 |

WAN 11

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:14::/64 | 2025:ACE:AD:14::1/64 | 2025:ACE:AD:14::2/64 |

WAN 12

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:15::/64 | 2025:ACE:AD:15::1/64 | 2025:ACE:AD:15::2/64 |

WAN 13

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:16::/64 | 2025:ACE:AD:16::1/64 | 2025:ACE:AD:16::2/64 |

WAN 14

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:17::/64 | 2025:ACE:AD:17::1/64 | 2025:ACE:AD:17::2/64 |

WAN 15

| Subnet Address | First Host Address | Second Host Address |
| --- | --- | --- |
| 2025:ACE:AD:18::/64 | 2025:ACE:AD:18::1/64 | 2025:ACE:AD:18::2/64 |

IPv6 Address Assignment

| DEVICE | INTERFACE | IPV6 ADDRESS | IPV6 ADDRESS2 |
| --- | --- | --- | --- |
| Multilayer Switch 5 | G1/0/21 | 2025:ACE:AD:A::1/64 | N/A |
|  | G1/0/22 | 2025:ACE:AD:B::1/64 | N/A |
|  | Vlan25 | 2025:ACE:AD:1::1/64 | N/A |
|  | Vlan35 | 2025:ACE:AD:2::1/64 | N/A |
| Multilayer switch 4 | G1/0/21 | 2025:ACE:AD:C::1/64. | N/A |
|  | G1/0/22 | 2025:ACE:AD:D::1/64 | N/A |
|  | Vlan25 | 2025:ACE:AD:1::1/64 | N/A |
|  | Vlan35 | 2025:ACE:AD:2::1/64 | N/A |
| LAN Distribution 1 | G0/0 | 2025:ACE:AD:A::2/64 | N/A |
|  | G0/1 | 2025:ACE:AD:D::2/64 | N/A |
|  | S0/0/0 | 2025:ACE:AD:E::1/64 | N/A |
|  | S0/0/1 | 2025:ACE:AD:F::1/64 | N/A |
| LAN Distribution 2 | G0/0 | 2025:ACE:AD:C::2/64 | N/A |
|  | G0/1 | 2025:ACE:AD:B::2/64 | N/A |
|  | S0/0/0 | 2025:ACE:AD:10::1/64 | N/A |
|  | S0/0/1 | 2025:ACE:AD:11::1/64 | N/A |
| Core Router 1 | G0/0 | 2025:ACE:AD:3::1/64 | N/A |
|  | G0/1 | 2025:ACE:AD:4::1/64 | N/A |
|  | S0/0/0 | 2025:ACE:AD:E::2/64 | N/A |
|  | S0/0/1 | 2025:ACE:AD:12::1/64 | N/A |
|  | S0/1/0 | 2025:ACE:AD:13::1/64 | N/A |
| Core Router 2 | G0/0 | 2025:ACE:AD:3::1/64 | N/A |
|  | G0/1 | 2025:ACE:AD:4::1/64 | N/A |
|  | S0/0/0 | 2025:ACE:AD:10::2/64 | N/A |
|  | S0/0/1 | 2025:ACE:AD:11::2/64 | N/A |
|  | S0/1/0 | 2025:ACE:AD:14::1/64 | N/A |
|  | S0/1/1 | 2025:ACE:AD:15::1/64 | N/A |
| Edge Distr Router 1 | S0/0/0 | 2025:ACE:AD:13::2/64 | N/A |
|  | S0/0/1 | 2025:ACE:AD:15::2/64 | N/A |
|  | S0/1/0 | 2025:ACE:AD:16::1/64 | N/A |
| Edge Distr Router 2 | S0/0/0 | 2025:ACE:AD:12::2/64 | N/A |
|  | S0/0/1 | 2025:ACE:AD:14::2/64 | N/A |
|  | S0/1/0 | 2025:ACE:AD:17::1/64 | N/A |
| WAN Router | S0/0/0 | 2025:ACE:AD:16::2/64 | N/A |
|  | S0/0/1 | 2025:ACE:AD:17::2/64 | N/A |
|  | S0/1/0. 123 | 2025:ACE:AD:18::1/64 | N/A |
|  | S0/1/0. 456 | 2025:ACE:AD:19::1/64 | N/A |
| RB Router | Fa0/0 | 2025:ACE:AD:5::1/64 | N/A |
|  | S0/0/0.321 | 2025:ACE:AD:18::2/64 | N/A |
| RS Router | Fa0/0 | 2025:ACE:AD:6::1/64 | N/A |
|  | S0/0/0.654 | 2025:ACE:AD:19::2/64 | N/A |
| PNA117V-PC1 | NIC | 2025:ACE:AD:1::10/64 | N/A |
| PNA117V-PC2 | NIC | 2025:ACE:AD:1::11/64 | N/A |
| NWP117V-PC1 | NIC | 2025:ACE:AD:2::10/64. | N/A |
| NWP117V-PC2 | NIC | 2025:ACE:AD:2::11/64 | N/A |
| Server1 | NIC | 2025:ACE:AD:3::10/64 | 2024:ABC:AD:14::5/64 |
| Server2 | NIC | 2025:ACE:AD:4::10/64 | 2024:ABC:AD:14::6/64 |
| RB-PC | NIC | 2025:ACE:AD:5::10/64 | N/A |
| RS-Server | NIC | 2025:ACE:AD:6::10/64 | N/A |

Topology

Show Vlan brief

Multilayer Switch5

Login details

User Access Verification

Password: cisco

MLSW5>enable

Password:class123

Multilayer Switch4

Login details

User Access Verification

Password: cisco

MLSW4>enable

Password:class123

Multilayer Switch3

Login details

User Access Verification

Password: cisco

MLSW3>enable

Password:class123

Multilayer Switch2

Login details

User Access Verification

Password: cisco

MLSW2>enable

Password:class123

RB Switch

RS switch

Show interface Vlan

Multilayer Switch5

Multilayer Switch4

Show interface Switchport

Multilayer Switch5

Show interface Switchport

Multilayer Switch4

Configurations screen dumps of the routers and L3 swatches

| Multilayer switch5 | Multilayer switch4 | Multilayer switch3 | Multilayer switch2 |
| --- | --- | --- | --- |
| MLSW5#show running-config Building configuration... Current configuration : 2076 bytes ! version 15.2 hostname MLSW5 no ip domain-lookup enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 ! (encrypted) service password-encryption ! ipv6 unicast-routing ! vlan 25 name PNA117V ! vlan 35 name NWP117V ! interface GigabitEthernet1/0/1 switchport mode access switchport access vlan 25 ! interface GigabitEthernet1/0/11 switchport mode access switchport access vlan 35 ! interface GigabitEthernet1/0/21 description Uplink to LAN-Dist1 no switchport ipv6 address 2025:ACE:AD:A::1/64 ipv6 ospf 15 area 0 ! interface GigabitEthernet1/0/22 description Uplink to LAN-Dist2 no switchport ipv6 address 2025:ACE:AD:B::1/64 ipv6 ospf 15 area 0 ! interface Vlan25 ipv6 address 2025:ACE:AD:1::1/64 ipv6 ospf 15 area 0 ! interface Vlan35 ipv6 address 2025:ACE:AD:2::1/64 ipv6 ospf 15 area 0 ! ipv6 router ospf 15 router-id 5.5.5.5 ! line con 0 ! password 7 0822455D0A16 ! Login ! exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA ! ! end | MLSW4#show running-config Building configuration... hostname MLSW4 no ip domain-lookup enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! vlan 25 name PNA117V ! vlan 35 name NWP117V ! interface GigabitEthernet1/0/21 description Uplink to LAN-Dist2 no switchport ipv6 address 2025:ACE:AD:C::1/64 ipv6 ospf 15 area 0 ! interface GigabitEthernet1/0/22 description Uplink to LAN-Dist1 no switchport ipv6 address 2025:ACE:AD:D::1/64 ipv6 ospf 15 area 0 ! interface Vlan25 ipv6 address 2025:ACE:AD:1::2/64 ipv6 ospf 15 area 0 ! interface Vlan35 ipv6 address 2025:ACE:AD:2::2/64 ipv6 ospf 15 area 0 ! ipv6 router ospf 15 router-id 4.4.4.4 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end | MLSW3#show running-config Building configuration... hostname MLSW3 enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! vlan 50 name Data-Center-1 ! interface FastEthernet0/1 switchport mode access switchport access vlan 50 ! interface GigabitEthernet0/1 description Uplink to Core1 no switchport ipv6 address 2025:ACE:AD:3::3/64 ipv6 ospf 15 area 0 ! interface Vlan50 ipv6 address 2025:ACE:AD:3::4/64 ipv6 ospf 15 area 0 ! ipv6 router ospf 15 router-id 3.3.3.3 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end | MLSW2#show running-config Building configuration... hostname MLSW2 enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! vlan 60 name Data-Center-2 ! interface FastEthernet0/11 switchport mode access switchport access vlan 60 ! interface GigabitEthernet0/1 description Uplink to Core1 no switchport ipv6 address 2025:ACE:AD:4::3/64 ipv6 ospf 15 area 0 ! interface Vlan60 ipv6 address 2025:ACE:AD:4::4/64 ipv6 ospf 15 area 0 ! ipv6 router ospf 15 router-id 2.2.2.2 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end |

| LAN Distribution 1 | LAN Distribution 2 | Core Router 1 | Core Router 2 |
| --- | --- | --- | --- |
| LAN-Dist1#show running-config Building configuration... hostname LAN-Dist1 enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! interface GigabitEthernet0/0 description Link to MLSW5 (WAN1) ipv6 address 2025:ACE:AD:A::2/64 ipv6 ospf 15 area 0 no shutdown ! interface GigabitEthernet0/1 description Link to MLSW4 (WAN4) ipv6 address 2025:ACE:AD:D::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/0 description Link to Core1 (WAN5) ipv6 address 2025:ACE:AD:E::1/64 ipv6 ospf 15 area 0 clock rate 64000 no shutdown ! interface Serial0/0/1 description Link to Core2 (WAN6) ipv6 address 2025:ACE:AD:F::1/64 ipv6 ospf 15 area 0 clock rate 64000 no shutdown ! ipv6 router ospf 15 router-id 11.11.11.1 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end | LAN-Dist2#show running-config Building configuration... hostname LAN-Dist2 enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! interface GigabitEthernet0/0 description Link to MLSW4 (WAN3) ipv6 address 2025:ACE:AD:C::2/64 ipv6 ospf 15 area 0 no shutdown ! interface GigabitEthernet0/1 description Link to MLSW5 (WAN2) ipv6 address 2025:ACE:AD:B::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/0 description Link to Core2 (WAN7) ipv6 address 2025:ACE:AD:10::1/64 ipv6 ospf 15 area 0 clock rate 64000 no shutdown ! interface Serial0/0/1 description Link to Core1 (WAN8) ipv6 address 2025:ACE:AD:11::1/64 ipv6 ospf 15 area 0 clock rate 64000 no shutdown ! ipv6 router ospf 15 router-id 22.22.22.2 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end | Core1#show running-config Building configuration... hostname Core1 enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! interface GigabitEthernet0/0 description Link to MLSW3 ipv6 address 2025:ACE:AD:3::1/64 ipv6 ospf 15 area 0 no shutdown ! interface GigabitEthernet0/1 description Link to MLSW2 ipv6 address 2025:ACE:AD:4::1/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/0 description Link to LAN-Dist1 (WAN5) ipv6 address 2025:ACE:AD:E::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/1 description Link to Core2 (WAN9) ipv6 address 2025:ACE:AD:12::1/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/1/0 description Link to Edge-Dist1 (WAN10) ipv6 address 2025:ACE:AD:13::1/64 ipv6 ospf 15 area 0 no shutdown ! ipv6 router ospf 15 router-id 1.1.1.1 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end | Core2#show running-config Building configuration... hostname Core2 enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! interface GigabitEthernet0/0 description Link to MLSW3 ipv6 address 2025:ACE:AD:3::2/64 ipv6 ospf 15 area 0 no shutdown ! interface GigabitEthernet0/1 description Link to MLSW2 ipv6 address 2025:ACE:AD:4::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/0 description Link to LAN-Dist2 (WAN7) ipv6 address 2025:ACE:AD:10::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/1 description Link to LAN-Dist2 (WAN8) ipv6 address 2025:ACE:AD:11::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/1/0 description Link to Edge-Dist2 (WAN11) ipv6 address 2025:ACE:AD:14::1/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/1/1 description Link to Edge-Dist2 (WAN12) ipv6 address 2025:ACE:AD:15::1/64 ipv6 ospf 15 area 0 no shutdown ! ipv6 router ospf 15 router-id 2.2.2.2 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end |

| Edge Distribution 1 | Edge Distribution 2 | WAN Router | RB Router | RS Router |
| --- | --- | --- | --- | --- |
| Edge-Dist1#show running-config Building configuration... hostname Edge-Dist1 enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! interface Serial0/0/0 description Link to Core1 (WAN10) ipv6 address 2025:ACE:AD:13::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/1 description Link to Core2 (WAN12) ipv6 address 2025:ACE:AD:15::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/1/0 description Link to WAN Router (WAN13) ipv6 address 2025:ACE:AD:16::1/64 ipv6 ospf 15 area 0 no shutdown ! ipv6 router ospf 15 router-id 10.10.10.1 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end | Edge-Dist2#show running-config Building configuration... hostname Edge-Dist2 enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! interface Serial0/0/0 description Link to Core1 (WAN9) ipv6 address 2025:ACE:AD:12::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/1 description Link to Core2 (WAN11) ipv6 address 2025:ACE:AD:14::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/1/0 description Link to WAN Router (WAN14) ipv6 address 2025:ACE:AD:17::1/64 ipv6 ospf 15 area 0 no shutdown ! ipv6 router ospf 15 router-id 10.10.10.2 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end | WAN-Router#show running-config Building configuration... hostname WAN-Router enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! interface Serial0/0/0 description Link to Edge-Dist1 (WAN13) ipv6 address 2025:ACE:AD:16::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/1 description Link to Edge-Dist2 (WAN14) ipv6 address 2025:ACE:AD:17::2/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/1/0 no ip address encapsulation frame-relay ipv6 address 2025:ACE:AD:18::1/64 ipv6 ospf 15 area 0 frame-relay map ipv6 2025:ACE:AD:18::2 123 broadcast frame-relay map ipv6 2025:ACE:AD:19::2 456 broadcast no shutdown ! ipv6 router ospf 15 router-id 70.70.70.1 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end | RB-Router#show running-config Building configuration... hostname RB-Router enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! interface FastEthernet0/0 description LAN Interface (Branch VLAN 70) ipv6 address 2025:ACE:AD:5::1/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/0.321 point-to-point description Frame Relay subinterface to WAN Router encapsulation frame-relay ipv6 address 2025:ACE:AD:18::2/64 ipv6 ospf 15 area 0 frame-relay interface-dlci 123 no shutdown ! ipv6 router ospf 15 router-id 50.50.50.1 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end | RB-Router#show running-config Building configuration... hostname RB-Router enable secret 5 $1$mERr$Vg8hQ/pc1ssw123 service password-encryption ! ipv6 unicast-routing ! interface FastEthernet0/0 description LAN Interface (Branch VLAN 70) ipv6 address 2025:ACE:AD:5::1/64 ipv6 ospf 15 area 0 no shutdown ! interface Serial0/0/0.321 point-to-point description Frame Relay subinterface to WAN Router encapsulation frame-relay ipv6 address 2025:ACE:AD:18::2/64 ipv6 ospf 15 area 0 frame-relay interface-dlci 123 no shutdown ! ipv6 router ospf 15 router-id 50.50.50.1 ! line con 0 password 7 0822455D0A16 login exec-timeout 5 30 ! line vty 0 4 password 7 0822455D0A16 login transport input ssh ! ip domain-name tut.ac.za crypto key generate rsa general-keys modulus 2048 username admin privilege 15 secret 5 $1$123abc$8slgA end |

Show ipv6 route

Frame Relay

ACLs Configuration

Ping Results

Communication between same VLAN and Different VLANs  initiated by PNA117V PCs and NWP117V PCs